[COLLEGE NAME]

[COLLEGE LOCATION / CITY]

[INSERT COLLEGE LOGO HERE]

[DEPARTMENT NAME]

BONAFIDE RECORD OF THE PROJECT WORK IN PARTIAL FULFILMENT

OF THE REQUIREMENTS FOR THE AWARD OF THE DEGREE OF

[DEGREE, e.g., BACHELOR OF ENGINEERING] IN [BRANCH OF STUDY]

DESIGN AND ASIC IMPLEMENTATION OF A

HEALTH-AWARE ARINC 429

COMMUNICATION CONTROLLER

Submitted by

[STUDENT NAME]     [REGISTER NUMBER]

Under the Guidance of

[FACULTY IN-CHARGE], [DESIGNATION]

[MONTH AND YEAR OF SUBMISSION]

BONAFIDE CERTIFICATE

This is to certify that the project report entitled “Design and ASIC Implementation of a Health-Aware ARINC 429 Communication Controller” is the bonafide record of the project work carried out by [STUDENT NAME] ([REGISTER NUMBER]), a student of [DEGREE] in [BRANCH OF STUDY], [DEPARTMENT NAME], [COLLEGE NAME], under my supervision and guidance during the academic year [ACADEMIC YEAR], in partial fulfilment of the requirements for the award of the degree of [DEGREE] in [BRANCH OF STUDY].

To the best of my knowledge, the work reported here has not been submitted to any other institution or university for the award of any other degree or diploma, and the results presented in Chapters 4 and 5 were obtained from actual simulations executed with Icarus Verilog 12.0.

Submitted for the university examination held on [DATE OF UNIVERSITY EXAMINATION].

| _______________________ Internal Guide | _______________________ Head of the Department |
| --- | --- |
| _______________________ [FACULTY IN-CHARGE] | _______________________ [DEPARTMENT NAME] |
| _______________________ Internal Examiner | _______________________ External Examiner |

ABSTRACT

Modern avionic systems rely on the ARINC 429 serial data bus for deterministic communication between line-replaceable units. This report documents Phase-I of a two-phase project whose final objective is the design and ASIC implementation of a health-aware ARINC 429 communication controller. Phase-I confines itself to the conventional, non-health-aware baseline: RTL design, functional verification, a fault-injection framework, and a machine-learning analysis pipeline; all health-monitoring hardware is deferred to Phase-II.

The conventional controller was implemented in Verilog-2001 and verified with three regression testbenches on Icarus Verilog 12.0, achieving pass rates of 9/9 (transmitter), 20/20 (receiver), and 15/15 (controller). A Review-2 fault-injection framework was then developed around the frozen baseline; a randomized campaign of 435 fault tests passed with all fourteen counter-consistency equations holding and produced a 46-column CSV dataset whose seed-1 instance contains 453 rows spanning fault classes 0–8 and 99. A leakage-controlled training protocol for six classifiers is defined, with the complete training code in Appendix C; the machine-learning and synthesis results are placeholders pending execution.

Keywords: ARINC 429; avionics; fault injection; RTL verification; machine learning; dataset; health monitoring; ASIC.

ACKNOWLEDGEMENT

I express my sincere gratitude to my project guide, [FACULTY IN-CHARGE], for the continuous guidance, detailed technical reviews, and encouragement provided throughout every stage of this project. The structured review-based schedule of this work, with one focused review per phase of activity, was instrumental in keeping the verification and documentation disciplined.

I am thankful to [HEAD OF DEPARTMENT NAME], Head of the Department of [DEPARTMENT NAME], and to our Principal, [PRINCIPAL NAME], for providing the laboratory facilities and the academic environment necessary for this work. I also thank the faculty and technical staff of the department for their timely support during the simulation and synthesis campaigns.

Finally, I thank my parents and friends for their patience and moral support during the course of this project, and I acknowledge the developers of the open-source Icarus Verilog simulator, Yosys, and the scikit-learn library, which were used extensively in this work.

[STUDENT NAME]

TABLE OF CONTENTS

CHAPTER 1   INTRODUCTION	5

1.1   Background and Motivation	5

1.2   Problem Statement	6

1.3   Objectives of Phase-I	6

1.4   Scope and Exclusions	7

1.5   Report Organization	8

CHAPTER 2   LITERATURE REVIEW	8

2.1   The ARINC 429 Bus in Commercial Avionics	8

2.2   Serial Bus Controller Design at Register-Transfer Level	8

2.3   Fault Injection as a Verification and Assessment Method	8

2.4   Machine Learning for Fault Diagnosis and Health Monitoring	9

2.5   Research Gap	9

CHAPTER 3   CONVENTIONAL ARINC 429 CONTROLLER DESIGN	9

3.1   Top-Level Architecture	9

3.2   Timing Constants	10

3.3   Word Format and Serial Bit Order	10

3.4   Operation of the Transmitter and Receiver	11

3.5   Supported Label Set	12

3.6   Functional Boundaries of the Conventional Design	12

CHAPTER 4   REVIEW-1: VERIFICATION, SYNTHESIS AND BASELINE FREEZE	12

4.1   Verification Strategy	12

4.2   Regression Results	13

4.3   Synthesis Plan and Result Template	14

4.4   Baseline Freeze	14

CHAPTER 5   REVIEW-2: FAULT-INJECTION FRAMEWORK	15

5.1   Framework Architecture	15

5.2   Fault Taxonomy	15

5.3   Campaign Configuration	16

5.4   Directed and Randomized Campaign Results	16

5.5   Counter Consistency and Dataset Integrity	17

5.6   Layered Honesty Semantics	17

CHAPTER 6   REVIEW-3: DATASET GENERATION AND MACHINE-LEARNING ANALYSIS	18

6.1   Dataset Description	18

6.2   Feature Schema	19

6.3   Task Definition and Leakage Prevention	20

6.4   Algorithms	20

6.5   Training Protocol and Code	21

6.6   Evaluation Metrics and Reported Results	22

CHAPTER 7   DISCUSSION AND PHASE-II DIRECTIONS	23

7.1   Key Findings and Lessons Learned	23

7.2   Limitations of Phase-I	24

7.3   Directions for Phase-II	24

CHAPTER 8   CONCLUSION	25

REFERENCES	25

APPENDIX A   RTL INTERFACE EXCERPTS	26

A.1   Top-Level Controller (arinc429_controller.v)	27

A.2   Transmitter FSM States (arinc429_tx.v)	30

A.3   Receiver FSM States (arinc429_rx.v)	31

APPENDIX B   46-COLUMN CSV SCHEMA	32

APPENDIX C   MACHINE-LEARNING TRAINING CODE	33

APPENDIX D   SIMULATION LOG EXCERPTS	52

D.1   Review-1 Regression Banners	52

D.2   Fault-Injection Campaign Configuration (mode1_s1.log, header)	56

D.3   Fault-Injection Campaign Summary (mode1_s1.log, final block)	58

Note: This Table of Contents is generated via field codes. To ensure page number accuracy after editing, please right-click the TOC and select “Update Field”.

LIST OF TABLES

| Table 3.1 Timing constants of the conventional ARINC 429 controller | [PAGE] |
| --- | --- |
| Table 3.2 Field map of the 32-bit ARINC 429 word | [PAGE] |
| Table 4.1 Review-1 functional regression results (Icarus Verilog 12.0) | [PAGE] |
| Table 4.2 Synthesis result template for the three DUT files | [PAGE] |
| Table 5.1 Fault-class taxonomy of the injection framework | [PAGE] |
| Table 5.2 Fault-injection campaign summary | [PAGE] |
| Table 6.1 Archived dataset files and row counts | [PAGE] |
| Table 6.2 Class distribution of the seed-1 dataset (453 rows) | [PAGE] |
| Table 6.3 Semantic groups of the 46-column CSV schema | [PAGE] |
| Table 6.4 Machine-learning algorithms and hyperparameters | [PAGE] |
| Table 6.5 Classification results of the six models [RESULT TO BE ADDED] | [PAGE] |
| Table 7.1 Phase-I versus Phase-II scope boundary | [PAGE] |

Note: Page numbers are to be confirmed after the actual figures are inserted and the final pagination is frozen.

LIST OF FIGURES

| Figure 3.1 Top-level block diagram of the conventional ARINC 429 controller | [PAGE] |
| --- | --- |
| Figure 3.2 Field layout and serial bit order of the 32-bit ARINC 429 word | [PAGE] |
| Figure 3.3 Transmitter FSM state sequence | [PAGE] |
| Figure 4.1 Review-1 regression waveform snapshot | [PAGE] |
| Figure 4.2 Synthesis top-level schematic of the controller | [PAGE] |
| Figure 5.1 Block diagram of the fault-injection verification framework | [PAGE] |
| Figure 5.2 Waveform of a representative fault-injection transaction | [PAGE] |
| Figure 6.1 Dataset generation flow from randomized fault-injection runs | [PAGE] |
| Figure 6.2 Class distribution of the seed-1 dataset | [PAGE] |
| Figure 6.3 Machine-learning training and evaluation pipeline | [PAGE] |
| Figure 7.1 Conceptual health-aware controller architecture for Phase-II | [PAGE] |

Note: Every figure box in this report is an explicit placeholder; the corresponding image (screenshot, diagram, or plot) is to be inserted by the author at the marked location before submission.

LIST OF ABBREVIATIONS AND SYMBOLS

| ARINC | Aeronautical Radio, Incorporated |
| --- | --- |
| ASIC | Application-Specific Integrated Circuit |
| CSV | Comma-Separated Values |
| DT | Decision Tree |
| EDA | Electronic Design Automation |
| FIFO | First-In, First-Out (buffer) |
| FIU | Fault-Injection Unit |
| FSM | Finite-State Machine |
| HDL | Hardware Description Language |
| KNN | K-Nearest Neighbours |
| LR | Logistic Regression |
| LSB | Least Significant Bit |
| LRU | Line-Replaceable Unit |
| ML | Machine Learning |
| MSB | Most Significant Bit |
| NN | Neural Network |
| NRZ | Non-Return-to-Zero |
| PAR | Place-and-Route |
| RF | Random Forest |
| RTL | Register-Transfer Level |
| SDI | Source/Destination Identifier |
| SSM | Sign/Status Matrix |
| SVM | Support Vector Machine |
| TB | Testbench |
| UUT | Unit Under Test |

# CHAPTER 1   INTRODUCTION

## 1.1   Background and Motivation

The ARINC 429 serial data bus is the dominant avionic data bus in commercial transport aircraft, connecting line-replaceable units (LRUs) over a twisted-shielded pair in a point-to-point topology in which one transmitter feeds up to twenty receivers [1]. The absence of arbitration makes worst-case latency directly analysable, which is why the bus has remained in service for decades. A conventional controller, however, has no means of observing the quality of its own communication: intermittent defects, ageing, or radiation-induced bit flips can degrade the transmit or receive path silently until the failure becomes operational. This motivates the health-aware communication controller, a design augmented with on-chip monitoring structures that signal an early warning when communication behaviour departs from its expected envelope.

## 1.2   Problem Statement

The long-term goal of the project is the design and ASIC implementation of a health-aware ARINC 429 controller, quantified against a conventional baseline in area, timing, power, and fault-monitoring capability. Such a comparison is only meaningful if the conventional baseline is first implemented, rigorously verified, and frozen, and if a labelled behavioural dataset exists from which data-driven monitoring can be developed. Phase-I therefore addresses three problems: the RTL design and complete functional verification of a conventional transmitter, receiver, and controller; the construction of a reproducible fault-injection framework with an explicit fault taxonomy and a machine-readable dataset; and the specification of a leakage-controlled machine-learning analysis pipeline, together with the synthesis preparation for the three design files.

## 1.3   Objectives of Phase-I

The objectives of Phase-I, each associated with one project review, are:

1. To study the ARINC 429 specification and extract the timing, framing, and parity rules a conforming controller must satisfy.

2. To design a conventional (non-health-aware) transmitter, receiver, and top-level controller in Verilog-2001.

3. To verify the design with three dedicated regression testbenches and freeze the baseline after Review-1.

4. To prepare the synthesis of exactly the three design files and define the result-recording template.

5. To build the Review-2 fault-injection framework with directed and randomized modes and reproducible seeds.

6. To generate a labelled CSV dataset and define a leakage-controlled training protocol for six classifiers.

7. To document all results honestly, separating obtained evidence from pending results.

Objectives 1 to 3 were completed and reviewed at Review-1, objectives 5 and 6 were delivered as framework and dataset at Reviews 2 and 3, and objective 4 is prepared but pending tool execution.

## 1.4   Scope and Exclusions

The scope of Phase-I covers the RTL of the three design modules, the three Review-1 testbenches, the fault-injection testbench and its simulation-only fault-injection unit, the archived CSV datasets, and the machine-learning protocol and code. All simulations were executed with Icarus Verilog 12.0 on a Linux host, and every quantitative claim cites its log or dataset file.

The exclusions are equally explicit. No FPGA implementation of any kind is included: no board, bitstream, pin constraints, or resource utilization. The physical electrical layer of ARINC 429 is out of scope; the RTL exposes a single-bit serial interface. No health-monitoring hardware exists in the Phase-I design and no machine-learning logic is embedded in the RTL; health awareness belongs exclusively to Phase-II. Wherever a result is pending, the text uses the marker [RESULT TO BE ADDED] or [SYNTHESIS RESULT TO BE ADDED], so that no unexecuted number can be mistaken for an obtained one.

## 1.5   Report Organization

Chapter 2 reviews the literature. Chapter 3 presents the conventional controller design, including what the design intentionally does not contain. Chapter 4 describes the Review-1 verification campaign, the synthesis plan, and the baseline freeze. Chapter 5 presents the Review-2 fault-injection framework and its evidence. Chapter 6 describes the dataset, the six machine-learning algorithms, and the anti-leakage training protocol with its complete code. Chapter 7 discusses findings and Phase-II directions, and Chapter 8 concludes. The appendices provide RTL excerpts, the full 46-column schema, the machine-learning training code, and simulation log excerpts.

# CHAPTER 2   LITERATURE REVIEW

## 2.1   The ARINC 429 Bus in Commercial Avionics

ARINC 429 defines a unidirectional point-to-multipoint serial bus carrying 32-bit words at 100 kbps or the low-speed 12.5 kbps option, with return-to-zero modulation on a differential pair [1]. Each word carries an eight-bit label that classifies the parameter, a Sign/Status Matrix (SSM), a Source/Destination Identifier (SDI), and odd parity over the whole word; the label is transmitted bit-reversed. Because there is no arbitration, the interword gap of at least four bit times bounds the word rate and makes the bus deterministic. These properties make ARINC 429 words self-describing and label-based filtering natural at the receiver.

## 2.2   Serial Bus Controller Design at Register-Transfer Level

Protocol-specific serial controllers form a mature RTL design class, built from a baud-rate generator, shift registers for serialization and deserialization, a sequencing finite-state machine (FSM), and status outputs; the relevant language semantics are defined by the IEEE Verilog standard [5]. An ARINC 429 controller differs from a plain UART chiefly in its word-level structure, its bit-reversed label order, and its strict interword gap. The literature on such controllers emphasizes that verification, not design, dominates the engineering effort, an observation that shaped this project: the verification and fault-injection chapters together occupy more space than the design chapter itself.

## 2.3   Fault Injection as a Verification and Assessment Method

Fault injection is the accepted experimental method for observing system behaviour under rare or dangerous conditions: faults are applied at the environment boundary according to a predefined fault model, and the response of the unit under test is compared with the expected behaviour. Model-based diagnosis research established the principle that deviations between observed and expected behaviour carry diagnostic information, which underlies the health-aware approach of Phase-II [6]. Three requirements follow for this project: an explicit fault taxonomy, seed-controlled reproducibility of randomized campaigns, and a strict separation of what the design under test detected from what the framework observed and what an external analysis concluded. The Review-2 framework of Chapter 5 implements all three.

## 2.4   Machine Learning for Fault Diagnosis and Health Monitoring

Machine-learning classifiers map observed behavioural features to diagnostic conclusions without an explicit analytical model. Decision trees learn interpretable if-then rules [8]; random forests aggregate decorrelated trees to reduce variance [2]; support vector machines construct maximum-margin boundaries suited to medium-sized tabular data [4]; the nearest-neighbour rule is the simplest instance-based control [3]; and neural networks trained by back-propagation model nonlinear decision surfaces [9]. Logistic regression completes the set as a linear baseline. All six are available in the scikit-learn library whose pipeline abstractions are used in the training code of Appendix C [7].

The diagnosis literature warns of two pitfalls that this project addresses directly. Data leakage arises when rows from the same transaction appear on both sides of the train-test split, inflating accuracy; the protocol of Section 6.5 therefore splits the data by transaction identity. Preprocessing leakage arises when scalers or imputers are fitted on the whole dataset before splitting; the code therefore fits every preprocessing step inside a pipeline fitted on the training partition only.

## 2.5   Research Gap

Public literature on ARINC 429 controllers rarely documents verification evidence at the depth needed to freeze a baseline with confidence, and seldom publishes labelled fault-injection datasets; the machine-learning diagnosis literature usually assumes a dataset rather than showing how to generate one from a specific design with reproducible seeds and an auditable schema. Phase-I addresses exactly this gap: a frozen conventional baseline, a seeded and schema-audited dataset, and a leakage-controlled learning protocol, forming the complete evidence chain that the Phase-II health-aware hardware requires for its quantified comparison.

# CHAPTER 3   CONVENTIONAL ARINC 429 CONTROLLER DESIGN

## 3.1   Top-Level Architecture

The conventional controller is a synchronous, single-clock design in Verilog-2001 organized as three modules: the transmitter arinc429_tx (188 lines), the receiver arinc429_rx (166 lines), and the top level arinc429_controller (84 lines). The top level instantiates both submodules and implements the receive-path source selection: on a receive request it latches either the internal transmit line or the external receive line according to the loopback setting, and releases the selection when a word is received. The interface exposes the transmit side (tx_start, label, sdi, data, ssm, tx_busy, tx_done), the receive side (rx_start, rx_word, rx_valid, parity_error, rx_busy), the serial lines arinc_tx and arinc_rx_ext, and loopback_enable; the port declarations are reproduced in Appendix A. All timing constants derive from the two parameters CLK_FREQ_HZ and ARINC_BAUD, so the same RTL is retargetable by parameter change alone, and every counter width is computed by a local clog2 helper rather than hardcoded. Figure 3.1 shows the block organization.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 3.1: Top-level block diagram of the conventional ARINC 429 controller

## 3.2   Timing Constants

The controller operates from a 50 MHz clock and drives the bus at the ARINC 429 high bit rate of 100 kbps, giving exactly 500 cycles per bit (one bit time of 10 microseconds). The transmitter holds the interword gap for 4 bit times (2000 cycles, 40 microseconds), a complete word occupies 16000 cycles (320 microseconds), and a full transaction occupies 18000 cycles (360 microseconds). Table 3.1 collects these constants; the receiver additionally resynchronizes at every transition using a half-bit window of 250 cycles. The fault-injection framework of Chapter 5 adopts the same constants and adds two project-defined thresholds of 22000 cycles (440 microseconds) each, described in Section 5.3.

Table 3.1: Timing constants of the conventional ARINC 429 controller

| Constant | Value | Meaning |
| --- | --- | --- |
| System clock frequency | 50 MHz (20 ns period) | Single synchronous clock domain |
| ARINC bit rate | 100 kbps | High-speed option of ARINC 429 |
| Cycles per bit | 500 cycles (10 us) | Derived as 50 MHz / 100 kbps |
| Word duration | 16000 cycles (320 us) | 32 bits serialized at 100 kbps |
| Interword gap | 2000 cycles (40 us) | 4 bit times between consecutive words |
| Transaction duration | 18000 cycles (360 us) | One word plus the interword gap |

## 3.3   Word Format and Serial Bit Order

Every word consists of 32 bits in five fields, summarized in Table 3.2. Bit 31 is the parity bit, computed as odd parity so that the XOR of all 32 transmitted bits is 1. Bits 30 to 29 carry the SSM, bits 28 to 10 the 19-bit data field, bits 9 to 8 the SDI, and bits 7 to 0 the label.

Table 3.2: Field map of the 32-bit ARINC 429 word

| Bits (word order) | Field | Width | Function |
| --- | --- | --- | --- |
| 31 | Parity | 1 | Odd parity over the 32-bit word (32-bit XOR = 1) |
| 30:29 | SSM | 2 | Sign/Status Matrix |
| 28:10 | Data | 19 | Data payload of the word |
| 9:8 | SDI | 2 | Source/Destination Identifier |
| 7:0 | Label | 8 | Word type identifier, transmitted bit-reversed |

The serialization order is a frequent source of implementation error and is therefore stated precisely: the transmitter sends the label first, then the SDI, the data field, the SSM, and finally parity. Within the label the bit order is reversed, so the byte stored as label[7:0] leaves the transmitter starting with label[7] and ending with label[0]. Figure 3.2 illustrates the field layout and serial order; both Review-1 testbenches contain directed checks for the bit-reversal rule.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 3.2: Field layout and serial bit order of the 32-bit ARINC 429 word

## 3.4   Operation of the Transmitter and Receiver

The transmitter is a three-state FSM (ST_IDLE, ST_TX, ST_GAP), shown in Figure 3.3. On an accepted tx_start it composes the 32-bit output register from label, SDI, data, and SSM, computes the odd-parity bit, reverses the label into serial order, and emits one bit every 500 cycles in NRZ form. After 32 bits it raises tx_done for exactly one clock, holds the 2000-cycle gap, and returns to idle; a start request while busy is ignored. The receiver is a four-state FSM (ST_IDLE, ST_FIRST, ST_RECEIVE, ST_DONE). Once armed by rx_start, it waits for the first falling edge, samples the bit centre at every 250-cycle half-bit boundary, shifts the bits into a 32-bit register, and on completion computes the 32-bit XOR, raises rx_valid with parity_error for one clock, and presents rx_word. Sampling at every edge lets the receiver tolerate frequency offsets between independent transmitters and receivers.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 3.3: Transmitter FSM state sequence

## 3.5   Supported Label Set

The application profile defines eight valid labels: 05, 15, 25, 35, 45, 55, 65, and 75 (hexadecimal). This set is an application definition, not a specification requirement. The verification environment uses it to exercise label-checking behaviour, and any injected label outside the set is recorded as an invalid-label observation; in the datasets of Chapter 6 the column label_supported records membership, and the archived datasets contain both supported values (5, 21, 37, 53, 69, 85, 101, 117 in the CSV decimal notation) and randomized values.

## 3.6   Functional Boundaries of the Conventional Design

For the honesty of the later comparison it is essential to state what the conventional design does not contain: the RTL has no FIFO buffer, no timeout detector, no label validator, and no bus-idle detector. These structures appear only as verification-level models inside the Review-2 testbench, where they define observation windows and classify injected conditions, and they are always reported as verification models rather than design features. The distinction matters for Phase-II, which will move selected monitoring structures into the RTL and measure their cost against this frozen baseline. Any statement that a fault was detected must be read in the layered sense of Section 5.6: detected by the DUT, observed at the verification level, classified by analysis, or declared an expected limitation because no such hardware exists.

# CHAPTER 4   REVIEW-1: VERIFICATION, SYNTHESIS AND BASELINE FREEZE

## 4.1   Verification Strategy

Each design module is verified by a dedicated self-checking testbench that derives its expected results from the specification rules independently of the implementation. The three testbenches contain 483, 466, and 598 lines of code respectively, are pure simulation models excluded from synthesis, and print a per-case verdict, summary counters, and a final OVERALL RESULT = PASS banner only when no check failed. All simulations ran on Icarus Verilog 12.0 under Linux, so every result is independently reproducible without licence constraints. The checks cover the timing constants of Table 3.1, the word format of Table 3.2, the label bit-reversal rule, and the odd-parity rule, so a complete pass constitutes conformance evidence for the specification subset of Chapter 3.

## 4.2   Regression Results

Table 4.1 summarizes the campaign. The transmitter testbench runs nine directed cases including reset behaviour, all-zero, all-one, and alternating words, a start-while-busy case, and reset during transmission. The receiver testbench performs twenty checks: nine single-bit faults were all detected and the single parity fault was detected, while the two two-bit faults were intentionally undetectable, which is the expected limitation of odd parity and not a defect. The controller testbench runs fifteen end-to-end transactions through the top-level multiplexer in loopback and external configurations, confirming the timing constants and the tx_done and rx_valid pulse widths.

Table 4.1: Review-1 functional regression results (Icarus Verilog 12.0)

| Testbench | Checks / cases | Result | Notable evidence from the logs |
| --- | --- | --- | --- |
| tb_arinc429_tx_review1_fix | 9 directed cases | 9/9 PASS | 0 timing failures, 0 gap failures, 0 tx_done width failures |
| tb_arinc429_rx_review1_fix | 20 checks | 20/20 PASS | 9/9 single-bit faults detected; 1/1 parity fault detected; 2 two-bit faults undetected (expected odd-parity limitation) |
| tb_arinc429_controller_review1_fix | 15 end-to-end checks | 15/15 PASS | CYCLES_PER_BIT = 500, GAP = 2000, TOTAL = 18000 confirmed; 0 width errors |

All 44 checks returned PASS with zero width and timing failures; the log banners are excerpted in Appendix D. The two two-bit escapes deserve emphasis: any even number of bit flips preserves odd parity, so these escapes are the arithmetically expected behaviour of the parity scheme, are labelled as expected limitations in the logs, and are not design defects. Figure 4.1 is reserved for a regression waveform snapshot showing, for example, the tx_done pulse and the serialized label bits of a transmitted word.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 4.1: Review-1 regression waveform snapshot

## 4.3   Synthesis Plan and Result Template

The synthesis unit is exactly the three design files of Chapter 3. The testbenches, the fault-injection unit, and the fault-injection testbench are simulation-only constructs containing delays, display statements, and file input/output with no hardware meaning; they are excluded by construction, not by tool option. The synthesis runs themselves are pending: Icarus Verilog is a simulator with no synthesis capability, and the dedicated runs with Yosys and the vendor tool of the target technology remain to be executed. Table 4.2 therefore defines the exact result template, with every cell a placeholder; no number from any FPGA report is admissible, because the project excludes FPGA implementation entirely.

Table 4.2: Synthesis result template for the three DUT files

| Metric | arinc429_tx | arinc429_rx | arinc429_controller |
| --- | --- | --- | --- |
| Target technology | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |
| Cell / LUT-equivalent area | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |
| Sequential cells / flip-flops | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |
| Longest path (setup) | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |
| Static power | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |
| Dynamic power (at 50 MHz) | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] | [SYNTHESIS RESULT TO BE ADDED] |

When the runs complete, each cell will cite the tool, version, constraints, and report file, and a synthesis log excerpt will join Appendix D. Until then the honest status of this chapter is: functional verification complete, synthesis pending. Figure 4.2 is reserved for the synthesis top-level schematic.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 4.2: Synthesis top-level schematic of the controller

## 4.4   Baseline Freeze

After Review-1 the three design files were frozen: all later work, including every fault-injection campaign of Chapter 5, uses the frozen files byte for byte. The freeze guarantees that the campaign results and datasets describe a single identified design, and it defines the reference for Phase-II: the health-aware controller will be reported as deltas against the frozen files using the template of Table 4.2. The frozen files were verified byte-identical to the originally uploaded sources before the campaigns began, so no change can silently invalidate the evidence chain.

# CHAPTER 5   REVIEW-2: FAULT-INJECTION FRAMEWORK

## 5.1   Framework Architecture

The Review-2 framework embeds the frozen baseline in an environment that corrupts the communication in controlled, classified, and reproducible ways. Its components are the frozen DUT, a simulation-only fault-injection unit, and the master testbench tb_arinc429_fault_injection.v, the corrected and extended version of the original Review-2 testbench, containing 2655 lines. The injector and testbench are strictly simulation helpers: they modify nothing inside the DUT, and all faults are applied at the environment boundary or through the stimulus. Three run modes are supported: Mode 0 (NORMAL) verifies that the framework does not perturb healthy traffic; Mode 1 (RANDOM) applies a seed-driven fault schedule at a configurable probability; Mode 2 (DIRECTED) applies a fixed script covering all timeout subtypes, gap-control cases, and boundary windows. Plusargs configure the seed, mode, test count, fault probability, error-rate window and threshold, timeout and idle thresholds, verification-model FIFO depth, and repeat-count range. Figure 5.1 shows the block organization.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 5.1: Block diagram of the fault-injection verification framework

## 5.2   Fault Taxonomy

Every injected condition belongs to one named class, recorded in the fault_class column; Table 5.1 gives the taxonomy, which is a project-defined analysis taxonomy rather than an ARINC construct. Class 20 is a subclass recorded in underlying_fault_type when a two-bit corruption escapes odd parity, class 98 is reserved for rows whose verification the framework could not establish (HEALTH_UNRELIABLE), and class 99 records reset-abort events. No archived dataset row carries class 98; no row carries class 20 as its primary class, because escaped two-bit corruptions are recorded under the parity class with the subclass flagged.

Table 5.1: Fault-class taxonomy of the injection framework

| Class | Name | Condition represented |
| --- | --- | --- |
| 0 | CLASS_NORMAL | Uncorrupted transaction; baseline health reference |
| 1 | CLASS_PARITY | Single-bit corruption on the serial stream, expected to raise parity_error |
| 2 | CLASS_TIMEOUT | No valid word within the timeout threshold (four subtypes) |
| 3 | CLASS_FIFO_OVF | Write to the verification-model FIFO when full (modelled, no RTL FIFO) |
| 4 | CLASS_FIFO_UNF | Read from the verification-model FIFO when empty (modelled, no RTL FIFO) |
| 5 | CLASS_INVALID_LBL | Label outside the supported application set of Section 3.5 |
| 6 | CLASS_REPEATED | Consecutive corrupted transactions within the repeat-count range |
| 7 | CLASS_BUS_IDLE | Bus idle longer than the idle threshold |
| 8 | CLASS_HIGH_RATE | Error rate within a window at or above the high-rate threshold |
| 20 | CLASS_TWO_BIT | Two-bit corruption subclass; escapes odd parity by arithmetic necessity |
| 98 | HEALTH_UNRELIABLE | Row whose verification could not be established by the framework |
| 99 | CLASS_RESET | Reset-abort event recorded as its own row |

## 5.3   Campaign Configuration

The randomized dataset campaign used a fault probability of 80 percent, an error-rate window of ten transactions, a high-error-rate threshold of 30 percent (project-defined), timeout and idle thresholds of 22000 cycles (440 microseconds) each, a verification-model FIFO depth of four, and a repeat-count range of three to six. Both thresholds sit intentionally above the normal 18000-cycle transaction so healthy traffic cannot trigger them; the log prints the configuration at start, and the same values appear in the idle_threshold and high_error_rate_threshold_pct columns of every row, making each dataset self-describing.

## 5.4   Directed and Randomized Campaign Results

Table 5.2 summarizes the executed campaigns. Every directed mode completed with all checks passing. The randomized campaign with seed 1 and 60 random tests expanded to 435 individual test cases, all passing, of which 247 were fault-bearing. Reproducibility was demonstrated in both directions: the same seed twice produced byte-identical logs and datasets, while seed 7 produced a different, self-consistent run, confirming seed-determined randomization.

Table 5.2: Fault-injection campaign summary

| Campaign | Configuration | Outcome |
| --- | --- | --- |
| Mode 0 (NORMAL) | default configuration | PASS; framework adds no perturbation to healthy traffic |
| Mode 2 (DIRECTED) | timeout subtypes 0-3; gap control; boundary windows 0/10/28/30/50% | PASS; all directed conditions classified as expected |
| Mode 1 (RANDOM), seed 1 | 60 random tests, 80% fault probability | PASS; 435/435 test cases; 14 consistency equations hold; 453-row dataset |
| Mode 1 (RANDOM), seed 42, run A and run B | identical plusargs | PASS; two runs byte-identical (reproducibility proof) |
| Mode 1 (RANDOM), seed 7 | identical plusargs except seed | PASS; differs from seed-42 run as expected (seed sensitivity) |
| Clamped configuration smoke | out-of-range plusargs (idle 100, timeout 5, threshold 0, window 1, depth 99, repeat 9..2) | PASS; 6 parameters clamped to legal ranges |

The campaign counters quantify the fault mix under seed 1: among corrupted-word cases, 63 landed in the data field, 27 in the label field, 8 in the SDI, 7 in the SSM, and 7 in the parity bit; of 11 windows crossing the high-error-rate evaluation, 7 were classified as high-rate windows and 4 recorded below threshold; 4 bus-idle and 4 reset events were exercised; and the verification-model FIFO recorded 38 writes and 40 reads with final occupancy zero. Figure 5.2 is reserved for a waveform excerpt of a representative fault-injection transaction.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 5.2: Waveform of a representative fault-injection transaction

## 5.5   Counter Consistency and Dataset Integrity

The testbench maintains five run-level counters (total transactions, valid transactions, errors observed, faults injected, fault tests) and enforces fourteen consistency equations tying these counters to the per-case verdicts and to the dataset rows, covering among others the pass-fail partition of the total counter, the agreement of the fault-test counter with the fault-bearing rows, the detection-status partition, and the expected-limitation accounting; the final log prints COUNTER CONSISTENCY: ALL 14 EQUATIONS HOLD, and any violation forces FAIL. Separately, an in-testbench row auditor checks every dataset row before it is written: strictly monotonic unique sample identifiers, legal ranges for row type, fault class, fault position, error rate, health label, and verification status, and coherent window counters (faulty transactions can never exceed window size). The summary reports dataset_integrity_failures = 0 (per-row CSV audit), and the counter gates the overall PASS. The seed-1 ledger closes exactly: 435 total tests with 247 fault tests, and 115 window/event rows flagged as expected limitations plus 17 per-event FIFO-model rows accounting for the expected-limitation total of 132.

## 5.6   Layered Honesty Semantics

The framework enforces four semantic layers that are never conflated. Layer one is what the DUT detected: credit is given only when the receiver itself raised parity_error, produced a mismatching word, or failed to produce the expected rx_valid pulse. Layer two is what the verification observed: facts established by the framework, such as timeout timing or modelled-FIFO occupancy. Layer three is what the analysis classified: repeated-error and high-error-rate windows are conclusions computed over several transactions, marked ANALYSIS_CLASSIFIED and printed with the word ANALYSIS. Layer four is the expected limitations: timeout, idle, FIFO, and label conditions cannot be detected by the DUT because the conventional RTL contains no such hardware, so those rows carry detection status EXPECTED_LIMITATION_NO_HW with the expected-limitation flag set.

The detection_status column encodes this layering with the values N/A, DUT_DETECTED, DUT_NOT_DETECTED, EXPECTED_LIMITATION_NO_HW, ANALYSIS_CLASSIFIED, and OBSERVED_AT_VERIFICATION_LEVEL, and health_label carries the ground-truth health family restricted to 0-8, with 98 reserved for framework-unresolvable rows. In the seed-1 dataset, 206 healthy rows carry N/A, 112 DUT_DETECTED, 115 EXPECTED_LIMITATION_NO_HW, and 20 ANALYSIS_CLASSIFIED; all 453 rows carry VERIFIED_PASS. This bookkeeping is what makes the dataset suitable for Chapter 6, because the target of a learned model can be chosen and interpreted unambiguously.

# CHAPTER 6   REVIEW-3: DATASET GENERATION AND MACHINE-LEARNING ANALYSIS

## 6.1   Dataset Description

Each campaign writes one CSV file named arinc429_ml_dataset_seed<SEED>.csv whose rows are typed records of one transaction, window, framework event, or reset event. Three datasets were archived, one per campaign seed, all sharing the fixed 46-column schema. The in-testbench row auditor guarantees schema conformance, and an external Python audit re-verified all three files independently, finding zero malformed rows, zero duplicate sample identifiers, and zero out-of-range values. Table 6.1 lists the archived files with their real row counts.

Table 6.1: Archived dataset files and row counts

| Dataset file | Seed | Rows | Columns | Campaign |
| --- | --- | --- | --- | --- |
| arinc429_ml_dataset_seed1.csv | 1 | 453 | 46 | Randomized, 60 tests (full campaign) |
| arinc429_ml_dataset_seed42.csv | 42 | 344 | 46 | Randomized (reproducibility pair, run A = run B) |
| arinc429_ml_dataset_seed7.csv | 7 | 347 | 46 | Randomized (seed-sensitivity run) |

The seed-1 dataset is the primary analysis dataset. Its rows are typed as 412 transaction rows, 20 window rows, 17 event rows, and 4 reset rows; all 453 rows carry verification status VERIFIED_PASS, and 115 rows carry the expected-limitation flag. Table 6.2 gives its class distribution, measured directly from the file; the imbalance is expected, because healthy and parity-class rows dominate a randomized campaign at the default fault mix. The target column health_label takes the values 0 to 8 (no row required code 98); its distribution matches Table 6.2 except that the four class-99 reset rows carry health_label 0, since a reset abort is not itself a communication health state. Figure 6.1 shows the dataset generation flow and Figure 6.2 is reserved for a class-distribution bar chart that the author can generate directly from the archived CSV.

Table 6.2: Class distribution of the seed-1 dataset (453 rows)

| fault_class | Meaning | Rows | Share of dataset |
| --- | --- | --- | --- |
| 0 | Normal (healthy) transaction | 206 | 45.5% |
| 1 | Parity (single-bit corruption) | 114 | 25.2% |
| 2 | Timeout | 13 | 2.9% |
| 3 | FIFO overflow (modelled) | 6 | 1.3% |
| 4 | FIFO underflow (modelled) | 7 | 1.5% |
| 5 | Invalid label | 83 | 18.3% |
| 6 | Repeated error window | 9 | 2.0% |
| 7 | Bus idle | 4 | 0.9% |
| 8 | High error rate window | 7 | 1.5% |
| 99 | Reset-abort event | 4 | 0.9% |

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 6.1: Dataset generation flow from randomized fault-injection runs

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 6.2: Class distribution of the seed-1 dataset

## 6.2   Feature Schema

The 46 columns fall into six semantic groups: four are excluded from model input, one column is the prediction target, and one group of eleven columns forms the behavioural feature set presented to the models. Table 6.3 summarizes the grouping, which drives both the training code of Appendix C and the leakage argument of Section 6.3; the name-by-name schema is reproduced in Appendix B.

Table 6.3: Semantic groups of the 46-column CSV schema

| Group | Columns | Role |
| --- | --- | --- |
| Prediction target | health_label (1) | Ground-truth health family; values 0-8 (98 reserved for unverifiable rows) |
| Identifiers | sample_id, seed, row_type, transaction_id, window_id, event_id, ref_transaction_id (7) | Bookkeeping only; used for grouping, never as features |
| Ground-truth and analysis flags | fault_class, underlying_fault_type, fault_position, word_bit_position, original_label, injected_label, label_supported, last_error_type, dominant_error_type, detection_status, verification_status, expected_limitation_flag, high_error_rate_condition (13) | Encode the injected truth or the framework verdict; excluded to prevent label leakage |
| Cumulative run counters | total_transactions, valid_transactions, errors_observed, faults_injected, fault_tests, parity_error_count, timeout_count, fifo_overflow_count, fifo_underflow_count, invalid_label_count, repeated_error_count, bus_idle_count (12) | Monotonic run-position proxies; excluded to prevent memorization of run order |
| Run constants | idle_threshold, high_error_rate_threshold_pct (2) | Constant within a run; no discriminative information |
| Behavioural features (model input) | parity_error, rx_word_mismatch, rx_valid, tx_done, idle_duration, consecutive_error_count, error_rate_pct, repeat_count, successful_transactions_between_errors, window_size, window_faulty (11) | Directly observable per-row behaviour; the model input |

## 6.3   Task Definition and Leakage Prevention

The learning task is: given the eleven behavioural features of a row, predict its health family health_label. This is multiclass classification over the health families (0 to 8, with 98 reserved for unverifiable rows). A model that succeeds using only observable behaviour, without access to the injected ground truth, demonstrates that the health state of the communication is recoverable from behaviour alone; that demonstration is the empirical premise on which the Phase-II health-monitoring hardware rests.

Two leakage channels must be closed. Column leakage: the dataset contains many columns that directly encode the injected truth or the framework conclusion, such as fault_class, last_error_type, and detection_status; including any would reduce the task to reading the answer key, so the eleven-feature whitelist of Table 6.3 closes this channel by construction. Partition leakage: consecutive rows describe overlapping segments of the same run, so random row-level splitting would place nearly identical rows on both sides of the split; the protocol therefore groups rows by transaction lineage (transaction, window, event, or sample identifier according to row type) and assigns whole groups to the test partition via GroupShuffleSplit, with GroupKFold applying the same discipline inside cross-validation. Preprocessing follows the same rule: a median imputer and a standard scaler are fitted inside a scikit-learn Pipeline, guaranteeing that they see the training partition only [7]. Class imbalance is handled by the class_weight option of the cost-sensitive models rather than by resampling, leaving the dataset untouched and auditable.

## 6.4   Algorithms

Six classifiers spanning the main algorithm families are defined with fixed hyperparameters so that the coming experiment is fully specified and reproducible. The set ranges from maximally interpretable to maximally expressive, because the Phase-II hardware mapping will favour models whose decision logic is small enough to implement in silicon. Table 6.4 records the exact configuration.

Table 6.4: Machine-learning algorithms and hyperparameters

| Model | Library class | Key hyperparameters |
| --- | --- | --- |
| Decision tree (DT) | tree.DecisionTreeClassifier | max_depth=8, min_samples_leaf=2, class_weight=balanced, random_state=42 |
| Random forest (RF) | ensemble.RandomForestClassifier | n_estimators=300, max_depth=12, class_weight=balanced, n_jobs=-1, random_state=42 |
| Logistic regression (LR) | linear_model.LogisticRegression | C=1.0, solver=lbfgs, max_iter=2000, class_weight=balanced |
| Support vector machine (SVM) | svm.SVC | kernel=rbf, C=10, gamma=scale, class_weight=balanced |
| K-nearest neighbours (KNN) | neighbors.KNeighborsClassifier | n_neighbors=5, weights=distance |
| Lightweight neural network (NN) | neural_network.MLPClassifier | hidden layers (32, 16), relu, alpha=1e-4, early_stopping, random_state=42 |

The decision tree is included for inspectability: its learned thresholds on features such as rx_word_mismatch or idle_duration can be read directly and can suggest hardware monitor points [8]. The random forest aggregates three hundred decorrelated trees for a variance-reduced reference [2]; the radial-basis support vector machine provides a maximum-margin nonlinear baseline [4]; the distance-weighted five-neighbour classifier serves as an instance-based control [3]; and the two-hidden-layer (32, 16) network trained by back-propagation with early stopping is intentionally small so that a future hardware mapping of its inference path remains realistic [9].

## 6.5   Training Protocol and Code

The complete training program is listed in Appendix C and delivered alongside this report as ml_train_arinc429.py. It loads one or more dataset files, applies the eleven-feature whitelist, derives the grouping column from the row type, performs a 20 percent group-wise holdout split with fixed random state 42, and trains all six models inside identical pipelines, additionally evaluating each with five-fold GroupKFold cross-validation on the training partition. The seed-42 and seed-7 files serve as external sanity checks, so a model trained on one campaign applies unmodified to data from another. The core of the protocol is reproduced below.

FEATURES = ['parity_error', 'rx_word_mismatch', 'rx_valid', 'tx_done',

'idle_duration', 'consecutive_error_count', 'error_rate_pct',

'repeat_count', 'successful_transactions_between_errors',

'window_size', 'window_faulty']

def make_pipe(model):

return Pipeline([('impute', SimpleImputer(strategy='median')),

('scale',  StandardScaler()),

('clf',    model)])  # fitted on TRAIN partition only

gss = GroupShuffleSplit(n_splits=1, test_size=0.20, random_state=42)

tr, te = next(gss.split(X, y, groups=group_ids))  # whole groups only

Figure 6.3 shows the end-to-end pipeline from the archived CSV files to the reported metrics.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 6.3: Machine-learning training and evaluation pipeline

## 6.6   Evaluation Metrics and Reported Results

Because Table 6.2 is strongly imbalanced, raw accuracy alone would overstate a model that simply predicts the majority health class. The protocol therefore reports, for every model: overall accuracy, balanced accuracy (the mean of per-class recalls), macro-averaged F1, per-class recall of every health family, and the full confusion matrix. The macro orientation judges a model on rare but diagnostically important classes such as bus idle or repeated-error windows, not merely on healthy traffic. The training runs have not been executed at the time of writing; in accordance with the no-fabrication policy, Table 6.5 reserves the result cells, and the confusion matrices will be added once the runs complete. The honest status is: dataset generated and audited, protocol fixed, code delivered, results pending.

Table 6.5: Classification results of the six models [RESULT TO BE ADDED]

| Model | Accuracy | Balanced accuracy | Macro F1 | Per-class recall |
| --- | --- | --- | --- | --- |
| Decision tree | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |
| Random forest | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |
| Logistic regression | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |
| Support vector machine | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |
| K-nearest neighbours | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |
| Lightweight neural network | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] | [RESULT TO BE ADDED] |

Once filled, the comparison answers the Phase-II planning question directly: if an interpretable model such as the decision tree approaches the black-box models on this feature set, the health-monitoring logic can be implemented as shallow threshold comparisons; if not, the lightweight network's inference path becomes the leading candidate for hardware mapping, and its synthesis cost will be measured against the frozen baseline of Chapter 4.

# CHAPTER 7   DISCUSSION AND PHASE-II DIRECTIONS

## 7.1   Key Findings and Lessons Learned

The first finding is procedural: verification effort dominated design effort by a wide margin, and the review schedule is what converted a working design into a frozen, evidence-backed baseline. The three Review-1 testbenches together contain more than three times the code of the design they verify, and every line of that surplus purchased a specific, auditable guarantee, from the tx_done pulse-width check to the label bit-reversal check. The second finding is that reproducibility is a designed property: the seed mechanism, once demonstrated by the byte-identical seed-42 pair, allowed every later statement about the campaigns to be made in the knowledge that a reader can regenerate the exact run.

The third finding concerns honesty semantics: separating, at the level of individual dataset columns, what the DUT detected, what the verification observed, what the analysis classified, and what must be declared an expected limitation. A concrete example is the timeout class: the conventional controller cannot time out because it has no timeout detector, so every timeout row is honestly flagged EXPECTED_LIMITATION_NO_HW rather than credited as a detection. This discipline preserved the meaning of every column and is, in the author's assessment, the property that will make the Phase-II comparison defensible. The fourth finding is that the two-bit escape of odd parity, observed in both the receiver regression and the campaigns, is an arithmetic property of the parity scheme, not a defect; recognizing it early prevented it from being misreported as a verification failure.

## 7.2   Limitations of Phase-I

Four limitations are stated explicitly. First, the synthesis runs are pending: the template of Table 4.2 is empty by design, so no hardware-cost statement can yet be made. Second, the six machine-learning models have not been trained; the protocol and code are complete, but Table 6.5 holds placeholders and no classification-performance claim is made. Third, the physical layer is absent by scope: the RTL operates on a single-bit serial interface, and the electrical robustness of the specification is inherited, not tested. Fourth, the dataset, while audited and reproducible, originates from a single design and one campaign configuration; the Phase-II comparison will benefit from additional campaign configurations and fault mixes. Each limitation maps onto a concrete Phase-II activity, and none undermines the results actually claimed: the functional pass of the frozen baseline, the correctness of the fault-injection bookkeeping, and the integrity of the archived datasets.

## 7.3   Directions for Phase-II

Phase-II will implement the health-aware controller and quantify it against the frozen baseline. The monitoring structures that Phase-I could only model at the verification level, namely the timeout and idle detectors, the label validator, and a bounded FIFO, will be moved into the RTL, and a compact health-monitor unit will aggregate their observations into a health status register visible to the host. The machine-learning analysis of Chapter 6 will be executed with the delivered code, and its outcome will select the on-chip monitoring form: shallow threshold logic if an interpretable model proves sufficient, or a small inference engine mapped from the lightweight network otherwise. The extended design will be synthesized with the same flow, file-list discipline, and result template as the baseline, so the deltas are measured, not estimated, and the fault-injection framework will be reused unchanged with the health-aware design as DUT to quantify the monitoring improvement on the identical taxonomy and campaigns. Table 7.1 fixes the boundary.

Table 7.1: Phase-I versus Phase-II scope boundary

| Aspect | Phase-I (this report) | Phase-II (planned) |
| --- | --- | --- |
| RTL content | Conventional controller only | Health-aware controller with monitoring hardware |
| Verification | Review-1 regression + Review-2 fault injection | Same framework, health-aware DUT |
| Dataset | Generated, audited, archived | Extended; reused for monitor validation |
| Machine learning | Protocol and code delivered; results pending | Executed; informs monitor logic |
| Synthesis | Template defined; runs pending | Executed for baseline and health-aware designs |
| FPGA | Excluded by scope | Excluded by scope (ASIC flow only) |

Figure 7.1 is reserved for the conceptual block diagram of the Phase-II health-aware controller, showing the monitoring structures and the health status register added to the frozen baseline.

[FIGURE PLACEHOLDER — INSERT ACTUAL IMAGE HERE]

Figure 7.1: Conceptual health-aware controller architecture for Phase-II

# CHAPTER 8   CONCLUSION

Phase-I set out to establish the complete evidence base on which a health-aware ARINC 429 controller can be built and quantified, and that objective has been met for every activity the phase defines. A conventional controller comprising a transmitter, a receiver, and a top-level multiplexer was implemented in Verilog-2001 with parameterized timing derived from the 50 MHz clock and the 100 kbps bit rate. Three self-checking regression testbenches verified the design against the word format, the label bit-reversal rule, and the odd-parity rule, returning complete pass rates of 9/9, 20/20, and 15/15 under Icarus Verilog 12.0, with the only undetected faults being the arithmetically expected escapes of two-bit corruptions from odd parity.

On the frozen baseline, a fault-injection framework with three run modes, a twelve-class taxonomy, and seed-controlled randomization was constructed and exercised. The full randomized campaign of seed 1 expanded to 435 test cases, all passing, with all fourteen counter-consistency equations holding and zero dataset-integrity failures; byte-identical reproduction was demonstrated for seed 42 and seed sensitivity for seed 7. The campaign archived a 46-column dataset whose seed-1 instance contains 453 fully audited rows spanning nine health families, together with two further seeds for external validation, and a leakage-controlled training and evaluation protocol for six classifiers was specified and delivered as executable code.

Two Phase-I activities remain open by design and are honestly marked: the synthesis of the three design files, for which the result template is prepared, and the execution of the machine-learning training runs, for which the metrics tables are prepared. All health-monitoring hardware, the execution of the learning pipeline, and the quantified comparison of area, timing, power, and fault-monitoring capability belong to Phase-II, which will reuse the frozen baseline, the fault-injection framework, and the datasets delivered here without modification.

# REFERENCES

[1] Aeronautical Radio, Inc., ARINC Specification 429: Mark 33 Digital Information Transfer System (DITS), Aeronautical Radio, Inc., Annapolis, Maryland, USA.

[2] L. Breiman, “Random Forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, 2001.

[3] T. M. Cover and P. E. Hart, “Nearest Neighbor Pattern Classification,” IEEE Transactions on Information Theory, vol. 13, no. 1, pp. 21–27, 1967.

[4] C. Cortes and V. Vapnik, “Support-Vector Networks,” Machine Learning, vol. 20, no. 3, pp. 273–297, 1995.

[5] IEEE, IEEE Standard for Verilog Hardware Description Language, IEEE Std 1364-2005, IEEE, New York, USA, 2005.

[6] R. Isermann, “Model-based Fault-Detection and Diagnosis – Status and Applications,” Annual Reviews in Control, vol. 29, no. 1, pp. 71–85, 2005.

[7] F. Pedregosa, G. Varoquaux, A. Gramfort, V. Michel, B. Thirion, O. Grisel, M. Blondel, P. Prettenhofer, R. Weiss, V. Dubourg, J. Vanderplas, A. Passos, D. Cournapeau, M. Brucher, M. Perrot, and E. Duchesnay, “Scikit-learn: Machine Learning in Python,” Journal of Machine Learning Research, vol. 12, pp. 2825–2830, 2011.

[8] J. R. Quinlan, “Induction of Decision Trees,” Machine Learning, vol. 1, no. 1, pp. 81–106, 1986.

[9] D. E. Rumelhart, G. E. Hinton, and R. J. Williams, “Learning Representations by Back-propagating Errors,” Nature, vol. 323, no. 6088, pp. 533–536, 1986.

# APPENDIX A   RTL INTERFACE EXCERPTS

The excerpts below reproduce, verbatim, the module declarations of the three frozen design files. They are provided so that the interfaces described in Chapter 3 can be checked against the sources; the complete files accompany this report in the project archive.

## A.1   Top-Level Controller (arinc429_controller.v)

module arinc429_controller #(

parameter integer CLK_FREQ_HZ    = 50000000,

parameter integer ARINC_BAUD     = 100000,

parameter integer INTERWORD_BITS = 4

)(

input  wire        clk,

input  wire        rst,

input  wire        tx_start,

input  wire [7:0]  label,

input  wire [1:0]  sdi,

input  wire [18:0] data,

input  wire [1:0]  ssm,

output wire        tx_busy,

output wire        tx_done,

input  wire        rx_start,

output wire [31:0] rx_word,

output wire        rx_valid,

output wire        parity_error,

output wire        rx_busy,

output wire        arinc_tx,

input  wire        arinc_rx_ext,

input  wire        loopback_enable

);

## A.2   Transmitter FSM States (arinc429_tx.v)

localparam integer CYCLES_PER_BIT =

(ARINC_BAUD > 0) ? (CLK_FREQ_HZ / ARINC_BAUD) : 0;

localparam integer GAP_CYCLES =

(INTERWORD_BITS > 0) ? (INTERWORD_BITS * CYCLES_PER_BIT) : 0;

localparam [1:0] ST_IDLE = 2'd0,

ST_TX   = 2'd1,

ST_GAP  = 2'd2;

## A.3   Receiver FSM States (arinc429_rx.v)

localparam integer CYCLES_PER_BIT =

(ARINC_BAUD > 0) ? (CLK_FREQ_HZ / ARINC_BAUD) : 0;

localparam integer HALF_CYCLES =

(CYCLES_PER_BIT > 1) ? (CYCLES_PER_BIT / 2) : 1;

localparam [1:0] ST_IDLE    = 2'd0,

ST_FIRST   = 2'd1,

ST_RECEIVE = 2'd2,

ST_DONE    = 2'd3;

# APPENDIX B   46-COLUMN CSV SCHEMA

The schema below applies identically to every row of every archived dataset file. Column numbers correspond to positions in the CSV header; the in-testbench row auditor (Section 5.5) enforces the ranges stated here on every row before it is written.

| No. | Column name | Meaning |
| --- | --- | --- |
| 1 | sample_id | Sequential row identifier; strictly monotonic and unique within a file (audited) |
| 2 | seed | User seed of the campaign that produced the row |
| 3 | row_type | 0=TRANSACTION, 1=WINDOW, 2=EVENT, 3=RESET |
| 4 | transaction_id | Communication event id; -1 when the row is not a transaction |
| 5 | window_id | Observation-window id; -1 on non-window rows |
| 6 | event_id | Framework event id; -1 on non-event rows |
| 7 | ref_transaction_id | Referenced transaction for window/event rows; -1 if none |
| 8 | fault_class | Ground-truth fault family per Table 5.1 (0-8, 20, 99) |
| 9 | underlying_fault_type | Finer mechanism, e.g. 20 = two-bit subclass; -1 if not applicable |
| 10 | fault_position | Serial bit position of the corruption (0-31); -1 if not applicable |
| 11 | word_bit_position | Word bit position corresponding to the serial position; -1 if N/A |
| 12 | original_label | Label value before injection (decimal); -1 if not applicable |
| 13 | injected_label | Label value after injection (decimal); -1 if not applicable |
| 14 | label_supported | 1 when the label belongs to the supported set {05,15,25,35,45,55,65,75} |
| 15 | total_transactions | Cumulative transaction counter at row creation time |
| 16 | valid_transactions | Cumulative rx_valid counter at row creation time |
| 17 | errors_observed | Cumulative observed communication error events |
| 18 | faults_injected | Cumulative faults intentionally introduced |
| 19 | fault_tests | Cumulative fault test-case counter |
| 20 | parity_error_count | Cumulative parity_error observations |
| 21 | timeout_count | Cumulative verified timeout conditions |
| 22 | fifo_overflow_count | Cumulative modelled FIFO overflow conditions |
| 23 | fifo_underflow_count | Cumulative modelled FIFO underflow conditions |
| 24 | invalid_label_count | Cumulative invalid-label conditions |
| 25 | repeated_error_count | Cumulative repeated-error case counter |
| 26 | bus_idle_count | Cumulative bus-idle conditions |
| 27 | idle_duration | Measured idle duration in cycles; -1 if not applicable |
| 28 | idle_threshold | Idle threshold in cycles (project-defined, constant within a run) |
| 29 | consecutive_error_count | Consecutive corrupted transactions at row time |
| 30 | error_rate_pct | Error rate within the current window, in percent |
| 31 | high_error_rate_threshold_pct | Project-defined high-rate threshold (constant within a run) |
| 32 | high_error_rate_condition | 1 when the high-rate condition is met in the window |
| 33 | repeat_count | Repeat count of the repeated-error case; 0 if not applicable |
| 34 | successful_transactions_between_errors | Clean transactions between two consecutive errors |
| 35 | last_error_type | Class code of the most recent error |
| 36 | rx_word_mismatch | 1 when the received word differs from the bit-accurate expected word |
| 37 | parity_error | 1 when the DUT raised parity_error on the row |
| 38 | rx_valid | 1 when the DUT pulsed rx_valid on the row |
| 39 | tx_done | 1 when the DUT pulsed tx_done on the row |
| 40 | window_size | Transactions in the current window; -1 on non-window rows |
| 41 | window_faulty | Faulty transactions within the window (0 <= value <= window_size) |
| 42 | dominant_error_type | Dominant error class of the window; -1 if not applicable |
| 43 | detection_status | 0=N/A, 1=DUT_DETECTED, 2=DUT_NOT_DETECTED, 3=EXPECTED_LIMITATION_NO_HW, 4=ANALYSIS_CLASSIFIED, 5=OBSERVED_AT_VERIFICATION_LEVEL |
| 44 | verification_status | 0=N/A, 1=VERIFIED_PASS, 2=VERIFIED_FAIL_DUT, 3=VERIFIED_FAIL_FRAMEWORK |
| 45 | expected_limitation_flag | 1 when the row records an expected limitation (no such RTL hardware) |
| 46 | health_label | Ground-truth health family 0-8; 98 = HEALTH_UNRELIABLE |

# APPENDIX C   MACHINE-LEARNING TRAINING CODE

The listing below is the complete training program whose protocol is described in Section 6.5. It is delivered alongside this report as ml_train_arinc429.py and runs with Python 3.8+ with pandas, numpy, and scikit-learn installed. The program has not been executed at the time of writing; when it is run, its printed metrics populate Table 6.5 of this report.

#!/usr/bin/env python3

# ml_train_arinc429.py - leakage-controlled training of six classifiers on the

# ARINC 429 fault-injection dataset (Phase-I, Review-3).

# Usage: python3 ml_train_arinc429.py arinc429_ml_dataset_seed1.csv

import argparse

import numpy as np

import pandas as pd

from sklearn.pipeline import Pipeline

from sklearn.impute import SimpleImputer

from sklearn.preprocessing import StandardScaler

from sklearn.model_selection import GroupShuffleSplit, GroupKFold

from sklearn.tree import DecisionTreeClassifier

from sklearn.ensemble import RandomForestClassifier

from sklearn.linear_model import LogisticRegression

from sklearn.svm import SVC

from sklearn.neighbors import KNeighborsClassifier

from sklearn.neural_network import MLPClassifier

from sklearn.metrics import (accuracy_score, balanced_accuracy_score,

f1_score, classification_report,

confusion_matrix)

RS = 42  # global random state for reproducibility

# Behavioural feature whitelist (Table 6.3): no identifiers, no injected

# ground truth, no cumulative run counters, no framework verdict columns.

FEATURES = [

'parity_error', 'rx_word_mismatch', 'rx_valid', 'tx_done',

'idle_duration', 'consecutive_error_count', 'error_rate_pct',

'repeat_count', 'successful_transactions_between_errors',

'window_size', 'window_faulty',

]

TARGET = 'health_label'

def load_data(paths):

frames = [pd.read_csv(p) for p in paths]

df = pd.concat(frames, ignore_index=True)

# -1 is the dataset sentinel for 'not applicable': treat as missing so

# that the median imputer (fitted on the train partition only) handles it.

df[FEATURES] = df[FEATURES].replace(-1, np.nan)

return df

def group_ids(df):

# Partition-leakage guard: group rows by transaction lineage. Window,

# event and reset rows group under their own identifiers.

gid = (df['row_type'].astype(str) + '_' +

np.select(

[df['row_type'].eq(0), df['row_type'].eq(1),

df['row_type'].eq(2)],

[df['transaction_id'], df['window_id'], df['event_id']],

default=df['sample_id']).astype(str))

return gid

def make_models():

return {

'Decision tree': DecisionTreeClassifier(

max_depth=8, min_samples_leaf=2, class_weight='balanced',

random_state=RS),

'Random forest': RandomForestClassifier(

n_estimators=300, max_depth=12, class_weight='balanced',

n_jobs=-1, random_state=RS),

'Logistic regression': LogisticRegression(

C=1.0, solver='lbfgs', max_iter=2000, class_weight='balanced'),

'Support vector machine': SVC(

kernel='rbf', C=10, gamma='scale', class_weight='balanced'),

'K-nearest neighbours': KNeighborsClassifier(

n_neighbors=5, weights='distance'),

'Lightweight NN': MLPClassifier(

hidden_layer_sizes=(32, 16), activation='relu', alpha=1e-4,

max_iter=500, early_stopping=True, random_state=RS),

}

def make_pipe(model):

# Imputer and scaler live INSIDE the pipeline: they are fitted on the

# training partition only and applied unchanged to the test partition.

return Pipeline([

('impute', SimpleImputer(strategy='median')),

('scale', StandardScaler()),

('clf', model),

])

def evaluate(name, y_true, y_pred, labels):

acc = accuracy_score(y_true, y_pred)

bal = balanced_accuracy_score(y_true, y_pred)

mf1 = f1_score(y_true, y_pred, labels=labels, average='macro',

zero_division=0)

print(f'=== {name} ===')

print(f'accuracy={acc:.4f}  balanced_accuracy={bal:.4f}  macro_f1={mf1:.4f}')

print(classification_report(y_true, y_pred, labels=labels,

zero_division=0))

print('confusion matrix (rows = true, cols = predicted):')

print(confusion_matrix(y_true, y_pred, labels=labels))

print()

return {'model': name, 'accuracy': acc, 'balanced_accuracy': bal,

'macro_f1': mf1}

def main():

ap = argparse.ArgumentParser()

ap.add_argument('csv', nargs='+', help='dataset CSV file(s)')

ap.add_argument('--holdout', type=float, default=0.20)

args = ap.parse_args()

df = load_data(args.csv)

groups = group_ids(df)

X, y = df[FEATURES], df[TARGET]

labels = np.sort(df[TARGET].unique())

# Group-wise holdout: whole transactions move together.

gss = GroupShuffleSplit(n_splits=1, test_size=args.holdout,

random_state=RS)

tr, te = next(gss.split(X, y, groups=groups))

X_tr, X_te = X.iloc[tr], X.iloc[te]

y_tr, y_te = y.iloc[tr], y.iloc[te]

print(f'rows={len(df)}  train={len(tr)}  test={len(te)}  groups_test='

f'{groups.iloc[te].nunique()}')

results = []

for name, model in make_models().items():

pipe = make_pipe(model)

pipe.fit(X_tr, y_tr)

results.append(evaluate(name, y_te, pipe.predict(X_te), labels))

# Robustness check: five-fold group-wise CV on the training partition.

gkf = GroupKFold(n_splits=5)

print('=== 5-fold GroupKFold (train partition) ===')

for name, model in make_models().items():

scores = []

for k_tr, k_te in gkf.split(X_tr, y_tr, groups=groups.iloc[tr]):

pipe = make_pipe(model)

pipe.fit(X_tr.iloc[k_tr], y_tr.iloc[k_tr])

pred = pipe.predict(X_tr.iloc[k_te])

scores.append(balanced_accuracy_score(y_tr.iloc[k_te], pred))

print(f'{name}: balanced_accuracy per fold = '

f'{[round(s, 4) for s in scores]}')

print('=== SUMMARY (holdout) ===')

for res in results:

print(f"{res['model']:24s} acc={res['accuracy']:.4f} "

f"bal={res['balanced_accuracy']:.4f} "

f"macro_f1={res['macro_f1']:.4f}")

if __name__ == '__main__':

main()

# APPENDIX D   SIMULATION LOG EXCERPTS

The excerpts below are copied verbatim from the archived simulation logs so that the numbers quoted in Chapters 4 and 5 can be traced to their source. Only the PASS banners and the summary blocks are reproduced; the complete logs accompany the project archive.

## D.1   Review-1 Regression Banners

-- tb_arinc429_tx_review1_fix (9 directed cases) --

PASS [RESET]

PASS [NORMAL] word=02af3505

PASS [MIXED] word=e48d16a5

PASS [ALL_ZERO] word=80000000

PASS [ALL_ONE] word=7fffffff

PASS [ALTERNATING] word=255555aa

PASS [BUSY_LATCH]

PASS [RESET_DURING_TX]

PASS [POST_RESET_RECOVERY] word=8048d288

Timing failures          = 0

Gap failures             = 0

tx_done width failures   = 0

OVERALL RESULT = PASS

-- tb_arinc429_rx_review1_fix (summary) --

Single-bit faults        = 9    Single-bit detected  = 9

Parity faults            = 1    Parity detected      = 1

Two-bit faults           = 2    Two-bit undetected   = 2

OVERALL RESULT = PASS

-- tb_arinc429_controller_review1_fix (summary) --

Two-bit undetected       = 1

Expected limitations     = 1

tx_done width errors     = 0    rx_valid width errors = 0

CYCLES_PER_BIT = 500  DATA_CYCLES = 16000  GAP_CYCLES = 2000

OVERALL RESULT = PASS

## D.2   Fault-Injection Campaign Configuration (mode1_s1.log, header)

ARINC 429 PHASE-1 FAULT INJECTION TESTBENCH (Review-2 rev B)

USER_SEED                 = 1  (same seed reproduces the identical

fault sequence)

RUN MODE                  = 1  (0=NORMAL, 1=RANDOM, 2=DIRECTED)

NUM_RANDOM_TESTS          = 60

FAULT_PROBABILITY         = 80 %

ERROR_RATE_WINDOW         = 10 transactions

HIGH_ERROR_RATE_THRESHOLD = 30 %   [PROJECT-DEFINED]

TIMEOUT_THRESHOLD         = 22000 cycles (440 us) [PROJECT-DEFINED]

IDLE_THRESHOLD            = 22000 cycles (440 us) [PROJECT-DEFINED]

FIFO_DEPTH (verification model) = 4  [NO FIFO IN RTL]

REPEAT_ERROR_RANGE        = 3..6

CYCLES_PER_BIT=500 GAP_CYCLES=2000 TOTAL_TX_CYCLES=18000

CSV dataset file = arinc429_ml_dataset_seed1.csv (46-column schema)

DUT files: UNMODIFIED Review-1 RTL; injector: TESTBENCH ONLY

## D.3   Fault-Injection Campaign Summary (mode1_s1.log, final block)

high_error_rate_windows_classified = 7 (ANALYSIS classification)

high_error_rate_windows_below      = 4

reset_tests                 = 4    reset_passed = 4

timing_failures             = 0

tx_done width failures      = 0    rx_valid width failures = 0

tb_internal_errors          = 0

consistency_violations      = 0

dataset_integrity_failures  = 0 (per-row CSV audit)

FIFO model: write_req=38 read_req=40 final_occ=0

COUNTER CONSISTENCY: ALL 14 EQUATIONS HOLD

OVERALL RESULT = PASS